# ECOAIR EXTRACTOR HOOD

# INSTRUCTIONS FOR USE

## BEFORE USING THE HOOD

- To get the best performance from the hood, please read these user instructions carefully. Keep this document for future reference.

- Packaging materials can be dangerous for children. Keep the packaging material (plastic bags, polystyrene, etc.) out of their reach.

- Make sure the hood has not been broken in transportation.

- Installation and connection to the mains and exhaust ducts must be carried out by a qualified technician, in accordance with the manufacturer's instructions and local safety regulations.

## USING THE HOOD

Switch on the hood a few minutes before starting to cook and leave it running for at least 5 minutes after you have finished.

Pull out the steam deflector (depending on model). Hood performance improves when the steam deflector is out.

Turn the speed selector to the required speed setting according to the quantity of fumes and steam to be extracted.

Switch off the hood 5 minutes after you have finished cooking.

Close the steam deflector (where fitted).

## SAFEGUARDING THE ENVIRONMENT

### Packaging

The packaging material is entirely recyclable, and marked with the recycling symbol. Follow local regulations for scrapping.

Packaging materials are potentially dangerous for children. Keep the packaging material (plastic bags, polystyrene, etc.) well out of their reach.

### Appliance

The appliance is made using recyclable materials. When scrapping, please respect local waste elimination procedures.

Make the appliance unusable by cutting off the power cord.

The appliance must be delivered to an authorized centre for scrapping.

## PRECAUTIONS AND GENERAL RECOMMENDATIONS

Always disconnect the hood by removing the plug or switching off the power supply before maintenance.

Exhaust air must not be conveyed through the same flue used by the heating system.

Do not flambé food under the hood since the flames generated could cause a fire.

Never leave a pan unattended when frying since cooking oil could ignite.

Regular maintenance guarantees smooth hood operation.

If the extractor hood is used together with a gas-burning hob or other appliance powered non-electrically, it is essential that the ambient negative pressure should not exceed a maximum of 4 pascals (4x10-5 bar).

For this reason, make sure the room is well ventilated.

Regularly remove deposits. Regularly clean and replace filters. Do not use inflammable material for outlets transporting exhaust air.

## CARE AND MAINTENANCE

Always disconnect the hood by removing the plug or switching off the power supply before maintenance.

### Grease filter

- The sponge grease filter should be washed in hot soapy water once a month (soak for 1-2 hours, do not wring. If necessary, repeat the process. Replace with a new filter every 5-6 washes).

- The grease filter paper should be replaced at least once a month.

- Grease filter paper with indicator should be replaced at least every two months or whenever the colour becomes visible through the holes of the grille. The paper filter is covered on one side.
The colour should not be visible through the grille holes when the filter is installed.

- The metal grease filter has an unlimited life and should be washed once a month by hand or in a dishwasher (60°C) with appropriate detergents.

- Leave the filter to dry before remounting it.

- Whenever cleaning or replacing the grease filters, clean all other exposed parts of the hood.

### Carbon filter

The carbon air filter cannot be washed or regenerated.

Replace the carbon filter (normal usage)

every six months 	(Rectangular carbon filter)

every four months 	(Circular carbon filter).

Replacing bulbs

- Pull out the steam deflector (depending on model)

- Open the grille or light fixture and remove the grease filter:

  - Pull back the white lever.

  - Twist the filter a quarter turn clockwise to unlock it and remove it.

- Change the burnt-out bulb: remove the old bulb and replace it with a new one, making sure that you use one of the same wattage.

- Put back the filter, close the grille or fixture and push in the white lever.

### Cleaning the hood

Warning! 	Failure to observe the instructions for cleaning the appliance and replacing or washing the filters may cause a fire.

Use a soft cloth with an alcohol-based product or a mild liquid detergent and never use abrasive substances.